### Table of Electric Load Curve Metrics & Indicators

| Indicator | Type of Indicator | Meaning | Formula / Calculation | Meaning in terms of Energy Behaviour | Pros & Cons | Sources |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Value (Peak Load) | Basic Statistic / Position Measure | Represents the highest power demand in the dataset over a given period. | $L_P = \frac{1}{n} \sum_{i=1}^n \max L_{i,j}$. | Reflects the user-dependent active share of consumption, marking periods of simultaneous high-power appliance usage. | Pros: Essential for sizing grid capacity and components. Cons: Reveals nothing about continuous, passive base loads. | Köhler et al. 2022; Li et al. 2018; Ihsane et al. 2018; Balasubramanian et al. 2021; Herraiz-Cañete et al. 2022; Dominguez et al. 2021; Hartvigsson & Ahlgren 2018; Dickert & Schegner 2011; Stevanato et al. 2025. |
| Minimum Value (Base Load) | Basic Statistic / Position Measure | Represents the lowest power demand occurring in the dataset. | N/A | Indicates the constant baseline energy footprint of devices running 24/7 (e.g., refrigerators) without active human intervention. | Pros: Excellent for identifying passive consumption. Cons: Misses dynamic human routines. | Köhler et al. 2022; Balasubramanian et al. 2021; Li et al. 2018; Hartvigsson & Ahlgren 2018. |
| Mean Load / Annual Consumption | Central Tendency | The average electrical power demand over a specific time span. | $L_M = \frac{1}{n} \sum \left( \frac{1}{48} \sum L_{i,j} \right)$. | Shows the overall magnitude of demand, correlating with household size, appliance ownership, and wealth. | Pros: Strong benchmark for comparing absolute consumption. Cons: Averages out timing, volatility, and peak behaviors. | Köhler et al. 2022; Balasubramanian et al. 2021; Zhang et al. 2022; Li et al. 2018; Ihsane et al. 2018; Herraiz-Cañete et al. 2022; Dominguez et al. 2021. |
| Relative Mean Power (Breakfast, Day, Dinner, Night) | Temporal Metric | Average consumption in a specific time block relative to total average consumption. | $\mu^R_i = \mu_i / \hat{\mu}$. | Identifies specific temporal habits, easily classifying user routines into specific active periods (e.g., "Night Consumers"). | Pros: Normalizes the scale, focusing purely on when people consume. Cons: Obscures the absolute peak demand magnitudes in Watts. | Kaur et al. 2022. |
| Modal Daily Peak Load Time ($T_P$) | Timing Metric | The half-hour interval that most frequently produces the daily maximum load. | $T_P = \text{mode} {j_{max} \L_{i,j_{max}} = \max L_{i,j}}$. | Directly indicates habitual peak routines, such as cooking dinner after work or waking up. | Pros: Highly useful for Time-of-Use (ToU) pricing and demand response. Cons: Doesn't indicate how intense the peak is. | Li et al. 2018. |
| Mean Daily Load Factor ($L_F$) | System / Performance | Ratio between average load and peak load. | $L_F = \frac{\text{Mean Load}}{\text{Peak Load}} \times 100$. | Describes the "peakiness" of a profile. Low load factors mean brief, intense spikes; higher factors suggest consistent, spread-out behavior. | Pros: Critical for assessing system efficiency and variability. Cons: Does not reveal when variations occur. | Li et al. 2018; Ihsane et al. 2018; Herraiz-Cañete et al. 2022; Dominguez et al. 2021; Hartvigsson & Ahlgren 2018; tpwrs.2020.3018936.pdf; Stevanato et al. 2025. |
| Capacity Factor | System Metric | The fraction of generated electricity compared to the maximum possible generation over a time frame. | $CF = \frac{\int_0^T P_L(t)dt}{P_G \cdot T}$. | Represents how well a system utilizes its capacity to meet aggregated electricity behaviours over time. | Pros: Primary indicator for evaluating the economic viability of mini-grid systems. Cons: Requires knowledge of system generation capacity. | Hartvigsson & Ahlgren 2018. |
| Coincidence Factor | System Metric | A measure of the likelihood that electric loads are used simultaneously by consumers. | $\frac{P_{L, Peak}}{P_{L, Tot}}$ | Reflects synchronized social routines; high coincidence means occupants activate heavy appliances at the exact same time. | Pros: Crucial for understanding the collective impact of behaviour on peak demand sizing. Cons: Requires accurate knowledge of the total installed load of all connected appliances. | Herraiz-Cañete et al. 2022; Hartvigsson & Ahlgren 2018. |
| Peak-to-Average Ratio (PAR) | System Metric | The ratio of peak power to average consumption. | $\frac{\text{Peak Load}}{\text{Average Load}}$ | A higher value means the ELP has more pronounced power peaks due to aggressive or concentrated appliance usage. | Pros: Helps target customers for demand reduction programs. Cons: Essentially the inverse of the Load Factor. | Köhler et al. 2022. |
| Peak Number Category (PNC) / ECPk | Shape / Feature | Classifies load patterns according to their number of electricity consumption peaks. | Count of peaks exceeding a designated threshold. | Translates directly to the number of distinct intensive activity periods mapping to active routines. | Pros: Highly intuitive for lifestyle segmentation and clustering. Cons: Sensitive to the arbitrary threshold defining a "peak". | Zhang et al. 2022. |
| V-Shape & M-Shape Features | Shape / Feature | Abstracted features capturing the morning and evening peaks. | Finding coordinates of $(T_{max}, C_{max})$ and $(T_{min}, C_{min})$. | Captures the abstracted macro-behavioral morning/evening energy routines over a simple parameter set. | Pros: Significantly reduces dimensionality compared to raw 24h variables. Cons: May miss anomalous midday appliance usage. | Al-Otaibi et al. 2016. |
| Load Blocks (Base, Off-peak, Transitional, Shoulder, Intermediate, Peak) | Temporal Categorization | Segments the 24h curve into discrete blocks based on load magnitude thresholds. | Categorization by hour/magnitude. | Maps specific times of day to intensity of human activity (e.g., active cooking vs passive sleep). | Pros: Very useful for Demand Side Management and tariff pricing. Cons: Rigid boundaries might miss shifting household schedules. | Balasubramanian et al. 2021. |
| Mean Relative Standard Deviation (MRSD) | Variability Measure | Averages the relative standard deviation across different time periods. | $\Delta\sigma = \frac{1}{4} \sum_{i=1}^4 \frac{\sigma_i}{\mu_i}$. | Measures the day-to-day variability and irregularity of a consumer (e.g., shift workers). | Pros: Captures true behavioural chaos and fluctuations well. Cons: Value can spike artificially if the mean consumption is close to zero. | Kaur et al. 2022. |
| Seasonal / Weekend vs Weekday Score | Contextual Metric | Difference in consumption between seasons or weekends/weekdays, proportional to average demand. | e.g., $WDScore = \sum \frac{\\mu_{WE,i} - \mu_{WD,i}\}{\mu_i}$. | Reflects the impact of climate changes or leisure/stay-at-home routines versus typical working day habits. | Pros: Isolates the effect of work schedules and climate on energy use. Cons: Requires a full year of data to compute. | Kaur et al. 2022. |
| Entropy of DECP | Complexity / Variability | Measures the diversity/complexity of Daily Electricity Consumption Patterns (DECP) within a dataset. | $Entropy(x) = -\sum p_c \log(p_c)$. | Indicates how regular or chaotic a household's daily routines are. High entropy means they utilize many different patterns. | Pros: Excellent for understanding the complexity of individual lifestyle stability. Cons: Requires pattern classification/dictionary building first. | Zhang et al. 2022. |
| Autocorrelation / Serial Correlation Coefficient ($r$) | Autocorrelation | Measures the correlation of the power consumption signal with a delayed copy of itself. | $r = \frac{cov(X_t, X_{t-\tau})}{\sigma_{X_t} \sigma_{X_{t-\tau}}}$. | Identifies cyclical behaviors or repeating daily routines across time lags. | Pros: Readily identifies periodic habits and self-similarity. Cons: Only tests for linear correlation. | Li et al. 2021; Ihsane et al. 2018. |
| Eta Correlation Ratio ($\eta^2$) | Statistical Dependency | Measures the relationship between dispersion within time-based demand and across total demand. | $\eta^2 = \frac{\sigma_{means}^2}{\sigma_{total}^2}$. | Determines exactly how much of the variation in electrical consumption depends specifically on the time of day. | Pros: Great for quantifying how heavily routines are tied to the clock. Cons: Doesn't explain why the time matters. | Ihsane et al. 2018. |
| InterQuartile Range (IQR) / Percentiles (P25, P50, P75) | Dispersion Measure | Measures the spread of the middle 50% of consumption data. | $Q3 - Q1$ | Defines "normal" daily operations vs. rare, highly intensive "unusual" demands (outliers). | Pros: Highly robust to extreme values and defines typical baseline usage. Cons: Ignores the chronological sequencing of the loads. | Köhler et al. 2022; Zhang et al. 2022; Li et al. 2018; Ihsane et al. 2018. |
| Fractal Dimension (FD) / Length of Curve | Complexity Measures | Geometric complexity indicating roughness or volatility of the load trajectory. | Estimated via mathematical estimators. | Captures structural complexity of behaviour. Reflects the natural stochastic spikiness of human interaction with appliances. | Pros: Exceptionally good at measuring the natural roughness of human behavior. Cons: Abstract concept; difficult to interpret in standard electrical units. | Köhler et al. 2022. |
| Sequence Dissimilarity / Distance (DTW, L2, LCSS) | Distance Measures | Measures exact shape matching or cost to align one sequence to another. | Sum of point-to-point costs (e.g. Euclidean). | Assesses how chronologically aligned or similarly shaped two occupants' daily lives are. | Pros: Standardized alignment of sequence shapes. Cons: Computationally expensive (especially DTW). | Zhang et al. 2022; Köhler et al. 2022. |
| RMSE / MAPE / NRMSE / NVF / MAE | Performance / Fit Measures | Evaluates the magnitude of deviation or exact matching error between measured and synthetic time series. | e.g. $\text{MAPE} = \frac{1}{n} \sum \\frac{y - \hat{y}}{y}\$ | Evaluates how well a modelled load curve replicates true behavioral variations. | Pros: Standardized metrics for evaluating model accuracy. Cons: Heavily penalizes realistic raw profiles simply for slight chronological shifts. | Köhler et al. 2022; Nti et al. 2020; Adeoye & Spataru 2019; Kewo et al. 2023; Ihsane et al. 2018; Dominguez et al. 2021; Widén et al. 2009. |
| MPDADA & MPDADMA | Forecast Error Metric | Mean Percentage Deviation Against Daily Average / Daily Moving Average. | $MPDADA = \frac{1}{K}\sum \frac{\P_k^r - P_k^f\}{\langle P_i \rangle}$. | Specifically evaluates how accurately a model predicts the peak demands compared to daily averages. | Pros: More sensitive to forecast errors during critical peak times than standard MAPE. Cons: Can be mathematically unstable if average daily load is extremely low. | Ihsane et al. 2018 |
| Blackout Frequency | Reliability | Shows how often the household exceeds the 89Wh battery limit. | Count of gaps > (Time Step + Buffer) |  |  |  |
| Mean Duration of Outage (Dout​) | Reliability | Measures the "recovery time" (usually until the next sunrise provides solar power). | Average time elapsed between last recorded log and next log. |  |  |  |
| Critical Discharge Window | Timing | Identifies if the blackout is caused by late-night lighting or early-evening high-power use. | The most frequent hour of the last log before a gap. |  |  |  |
| Reliability Index (RI) | System | Quantifies the percentage of time the user actually has energy access. | Total Theoretical HoursActual Recorded Hours​×100. Measures the gap between "Desired" and "Actual" consumption. |  |  |  |
| Energy Sufficiency Gap | System | Explains if "aggressive" practices lead to longer periods without light | Correlation between Peak Load (LP​) and subsequent Blackout duration. |  |  |  |
| Climatic Blackout Rate | Contextual | Introduces "Weather Stochasticity" into the demand model. | System failure caused by low v_pv due to clouds/rain. |  |  |  |
| Behavioral Blackout Rate | Practice | Validates "Aspirations" vs. "Material" constraints. | System failure caused by high usage (e.g., charging for neighbors). |  |  |  |